# Purpose

Detail how special processes—those where the resulting output cannot be fully verified by subsequent inspection—are qualified, controlled, and periodically re-validated, ensuring continued conformity with AS9100 Clause 8.5.1.2.

# Scope

Covers plating, anodizing, soldering, adhesive bonding, heat-treat, welding, conformal-coating, and any other process where deficiencies may only appear after the product is in service.

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Process Engineer | Defines qualification plan, acceptance criteria, and control parameters; maintains approved supplier list for outsourced special processes. |
| Production Supervisor | Ensures shop-floor work instructions reflect current qualified parameters; prevents use of unqualified operators or settings. |
| Qualified Operator / Technician | Executes the process exactly to qualified parameters; records run data; identifies any deviations. |
| Quality Control (QC) Inspector | Verifies test coupons, witness samples, and process charts; initiates re-qualification when limits are exceeded. |
| Quality Manager | Reviews qualification evidence, approves periodic re-qualification schedule, and escalates systemic issues to corrective action. |

# Definitions

| Term | Definition |
| --- | --- |
| Special Process | Process where conformity cannot be fully verified by subsequent testing or inspection, so it must be continuously controlled. |
| Process Qualification Coupon | Representative sample processed alongside production parts to verify parameters. |
| Critical Parameter | Variable that has a direct effect on final product conformity (e.g., bath temp, time, torque, oven profile). |
| Periodic Re-qualification | Scheduled demonstration—specified interval or lot—showing the process still meets acceptance criteria. |

# Work Instruction

| Step | Action | Responsible | Record |
| --- | --- | --- | --- |
| 1 | Identify Special Process – During DFMEA / PFMEA, flag operations where final conformity cannot be verified by inspection alone. | Process Eng. | PFMEA |
| 2 | Develop Qualification Plan – Define critical parameters, test coupons, sample size, and acceptance criteria; document in the Process Control Plan. | Process Eng. | Process Control Plan |
| 3 | Initial Qualification Run – Produce coupons under defined parameters; perform destructive or NDT per spec; record results. | Qualified Operator / QC | Qualification report |
| 4 | Approve Process Parameters – If results meet criteria, list qualified settings, tooling, and operator cert level in controlled Special-Process Work Instruction. | QC & Process Eng. | Qualified parameter sheet |
| 5 | Operator Certification – Train operators; log practical and written test results; add to Competence Matrix (A-720-001). | Quality & Training | Training records |
| 6 | Production Control – Run each batch using only qualified parameters; record critical values on Process Run Sheet or automated data logger. | Operator | Run sheet / data file |
| 7 | Witness Samples / Coupons – For each lot (or as defined), process coupons; QC inspects per qualified method; attach results to batch traveller. | Operator / QC | Coupon results |
| 8 | Review of Control Charts – QC monitors SPC charts or data logs; if any parameter drifts outside control limits, stop and re-qualify. | QC Inspector | SPC chart |
| 9 | Periodic Re-qualification – At interval (e.g., 1 year or 200 lots) perform full qualification test; update records. | QC & Process Eng. | Re-qualification report |
| 10 | Non-conformance Handling – Deviations trigger NCR per WI-870-001; affected product quarantined until MRB disposition. | QC | NCR |
| 11 | Supplier Control – External special-process providers must hold equivalent accreditation (e.g., Nadcap) or pass Supplier Qualification (WI-840-001). | Supply Chain / QC | F-840-001 |
| 12 | Change Management – Any change in chemistry, equipment, software, or operator requires step 3 re-qualification before release. | Process Eng. | DCR (F-750-002) |

# Review & Update Frequency

• Annual review, or immediately after process change, customer requirement update, or audit finding.

# References

• AS9100 Rev D Clause 8.5.1.2 – Validation & Control of Special Processes
• P-851-01 Validation of Special Processes (Procedure)
• WI-840-001 Conducting Supplier Qualification
• WI-870-001 Non-conformance Processing
• P-750-01 Control of Documented Information

# Documented Information (maintained separately)

• Process Qualification Plan & reports
• Operator certification records (Competence Matrix)
• Process Control Plan & SPC charts / data logs
• Re-qualification schedules and results
• NCRs and associated MRB dispositions

# Record of Review

| Review Date | Reviewed By | Summary of Changes | MRM Ref. |
| --- | --- | --- | --- |
| – | – | Initial release | – |