# Purpose

Describe how products are identified and traced from receipt to shipment so that serial / lot history, inspection status, and configuration are always known, meeting AS9100 Clause 8.5.2.

# Scope

Applies to raw materials, in-process parts, finished goods, customer-supplied items, and repair returns handled at any [The Organization] facility or approved external provider.

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Stores Clerk | Applies incoming ID labels; records lot/heat numbers in ERP. |
| Operator / Technician | Transfers IDs to sub-assemblies; records serial/lot on travellers and routers. |
| Production Supervisor | Verifies ID integrity during shift audits; prevents mixing of lots. |
| Quality Control (QC) Inspector | Confirms ID/trace data at each inspection point; stamps travellers “Accepted” or “Rejected.” |
| Document Control | Maintains label templates and ID formats; archives traceability records. |

# Definitions

| Term | Definition |
| --- | --- |
| Unique Identifier (UID) | Serial number or lot number that is never reused for that part family. |
| Split Lot / Kit | A parent lot divided into daughter lots for separate processing or shipment. |
| Status Marking | Visual indicator (stamp, tag, barcode) showing inspection status (Accepted, Rework, Reject). |

# Work Instruction

| Step | Action | Responsible | Record |
| --- | --- | --- | --- |
| 1 | Incoming Material – Label each container with part no., lot/heat, quantity, and receipt date; scan into ERP location. | Stores Clerk | ERP receipt |
| 2 | Assign Serial Numbers – For serial-controlled parts, generate UID in ERP; engrave, laser-mark, or label per drawing note. | Operator | Traveller |
| 3 | Transfer IDs During Assembly – Copy parent serial/lot to sub-assembly traveller; affix temporary tag if part is too small for direct mark. | Operator | Traveller |
| 4 | Maintain Traveller – Record operation, date, operator ID, and inspection stamp at each routing step. | Operator & QC | Traveller |
| 5 | Status Marking – Use green stamp “ACCEPTED” for pass, red “REJECTED” for NCR; if Rework, attach yellow tag with NCR number. | QC Inspector | Traveller / Tag |
| 6 | Split Lots – When dividing a lot, create daughter lot numbers in ERP, update both travellers, and segregate physical bins. | Production Supervisor | ERP split record |
| 7 | Configuration Tracking – For rev-controlled parts, verify drawing / BOM revision matches traveller revision before processing. | Operator | Traveller |
| 8 | Customer-Supplied Items – Apply blue “Customer Property” tag, record UID and condition on Customer Property Register (F-850-003). | Stores Clerk | F-850-003 |
| 9 | Repair / Return Items – Assign RMA number; tag as “Return”; record original serial and RMA in ERP. | Service Coord. | SWO |
| 10 | Final Verification – Before shipment, QC checks packaging labels match CoC: part no., rev, serial/lot, quantity. | QC Inspector | CoC & shipping label |
| 11 | Archive Records – Scan signed travellers / routers; store ERP extract of serial & lot genealogy for ≥ 10 yrs (Retention Cat. A). | Document Control | DMS archive |

# Review & Update Frequency

• Annual review or upon change in part-marking method, ERP system, or customer ID requirements.

# References

• AS9100 Rev D Clause 8.5.2 Identification & Traceability
• P-853-01 Identification & Traceability (Procedure)
• WI-850-001 Production Execution & Control
• WI-870-001 Non-conformance Processing
• P-750-01 Control of Documented Information

# Documented Information (maintained separately)

• ERP serial / lot genealogy reports
• Label templates and stamping matrices
• Signed travellers / routers with inspection stamps
• Customer Property Register F-850-003
• NCR tags and rework labels

# Record of Review

| Review Date | Reviewed By | Summary of Changes | MRM Ref. |
| --- | --- | --- | --- |
| – | – | Initial release | – |