# Purpose

Describe how finished products are inspected, certified, and released—including split-tag approval—so every shipment meets customer, regulatory, and AS9100 Rev D Clause 8.6 requirements.

# Scope

Applies to all production and repair items shipped from any [The Organization] site or released to customers.

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Quality Control (QC) Inspector | Perform final inspection, verify documentation, complete release tag/coC. |
| Production Supervisor / Repair Supervisor | Present product and traveller to QC; ensure rework/NCRs are closed. |
| Quality Manager | Approves split-tag usage, maintains authorised signatories list; audits release records. |
| Document Control | Archives signed tags/CoC in controlled system; links to shipment record. |

# Definitions

| Term | Definition |
| --- | --- |
| CoC | Certificate of Conformance issued to non-FAA customers (template F-860-003). |
| Traveller | ERP or shop packet with all in-process inspections completed and signed. |

# Work Instruction

| Step | Action | Responsible | Record |
| --- | --- | --- | --- |
| 1 | Final Traveller Review – Confirm all process and hold-point signatures are complete; verify NCR numbers are closed in ERP. | QC Inspector | Traveller / ERP |
| 2 | Dimensional & Visual Inspection – Inspect per drawing and customer specs; sample per inspection plan or 100 % for safety-critical parts. | QC Inspector | Inspection sheet |
| 3 | Functional / Electrical Test (if required) – Perform acceptance test procedure; attach data printout. | QC Inspector / Test Tech | Test report |
| 4 | Document Verification – Ensure latest drawing revision, special-process certs, material certs, and traveller match part/serial. | QC Inspector | Doc packet |
| 5 | Prepare Certificate of Conformance (CoC) using template F-860-003; list part(s), revision, quantity, serial/lot, and contract reference. | QC Inspector | CoC |
| 6 | Authorised Signature – QC Inspector (listed on authorised signatory sheet) signs and dates release document. | QC Inspector | Release doc |
| 7 | Attach Labels & Tags – Affix serial number label, conformity tag, and handling symbols to product/pack per WI-853-001. | QC Inspector | — |
| 8 | Archive & Issue Copy – Scan signed release doc; save in ERP document vault; provide copy to Logistics and Production. | Document Control | ERP |
| 9 | Ship Product – Logistics packs product using WI-854-001; include release doc and packing list with shipment. | Logistics | Shipping record |
| 10 | Record KPI & Close WO – QC logs First-Pass Yield and release approval time; Production closes WO in ERP. | QC / Production | KPI dashboard |

# Review & Update Frequency

• Annual review, or when FAA / customer tag requirements change.
• Immediate update if authorised signatory list changes.

# References

- AS9100 Rev D — Clause 8.6 Release of Products and Services

- P-860-01 Release of Products & Services (Procedure)

- WI-853-001 Identification & Traceability

- WI-854-001 Preservation & Packaging Methodology

- P-750-01 Control of Documented Information

# Documented Information (maintained separately)

• 8130-3 or 8130-3ST release certificates
• Certificate of Conformance template F-860-003
• Completed travellers and inspection/test reports
• Authorised signatory sheet
• KPI dashboard (release metrics)

# Record of Review

| Review Date | Reviewed By | Summary of Changes | MRM Ref. |
| --- | --- | --- | --- |
| – | – | Initial release | – |