# Purpose and Scope

This process identifies the requirements of In-Process Inspections for cut material.

# Terminology

| Inventory Disposition Form (ID001) | The Inventory Disposition Form is used to adjust inventory out of prime stock to rems, scrap, etc. |
| --- | --- |
| Dimensionally | Measurement of width, length, and /or height per customer specifications. |

# Process Responsibilities

| Warehouse Machine Saw Operators | - Reviews Work Orders and processes material according to the customer specifications. - Pulls appropriate material or verifies the material pulled by others matches the order requirements. Confirm grade, alloy, size, quantity, and Heat/Lot stencil (if available). - Confirms material pack/heat documented on Work Order is available in the Business System before cutting. - Periodically inspect processed cuts, to ensure they are within tolerance. - Records any nonconforming material on the Inventory Disposition Form (ID001). - Verifies the total quantity cut, completes, and signs the Work Order as the Operator. |
| --- | --- |

# Procedure Statements

- The Machine/Saw Operator verifies the alloy, grade, dimension, and condition of material against the Work Order specifications and records the Pack number, Lot/Heat Number, and verifies number of pieces on the Work Order.

- When required, the Operator will dimensionally inspect the first, middle, and last cut pieces. When bundle cutting, will randomly measure pieces from the top, middle, and bottom of the bundle cut.

- When checking that parts meet requirements use the operator notes area to record any notes.

- Operator certification is sufficient to show requirements compliance.

- Statistically monitor compliance by categorizing and monitoring Credit Notes or complaints.

Quality Expectations

- Associates shall not knowingly provide poor quality material to a customer.

- Associates will be diligent in their attention to detail and not fulfill any order with blemished, gouged, scratched, rusted, bent, or damaged material and will consult Supervisors/Managers to confirm.

- Items that may be acceptable per specification, but do not meet the visual expectations of the inspector will be held until a customer concession is obtained.

- IMS will always strive to only fulfill customer orders with prime quality, mill finish material through appropriate and protective handling.

- Material that is cut to size will incur some minor processing evidence in appearance which is common and accepted in the industry.

- As a Quality certified distributor, and to ensure prime quality of our stock, we make every attempt to ‘handle’ items as little as possible in order to give our customers as close to a Mill-direct experience possible.

- Associates will maintain accuracy of traceability on product by verifying correct Pack, Heat and Mill Test Report records.

# Risks and Mitigations

| Risks | Mitigations |
| --- | --- |
| Verification of changes to customer orders e.g., change notices, amendments, etc. | Inspection and verification using the correct specs, acceptance criteria, and equipment. |
| Using an out-of-tolerance tool or instrument. | Establishment of criteria for acceptance of products (measurement, monitoring, inspection, verification, and validation). |
| Spurious readings on equipment (imprecision). | A well-maintained Preventative Maintenance program, accompanied by knowledgeable staff Mechanics. |

# Reference and Applicable Documents

Organizational Quality Processes Manual (QSM-9000)

Product Identification (QSP 8.1)

Inventory Disposition Form (ID001)

# Personnel to be Trained*

Warehouse & Store Operations Staff

Sales*

Branch & Store Management*

*Personnel that are determined to be impacted by this process shall require training in its effective implementation.

# Revisions Table

| Date | Description of Change | Approved by |
| --- | --- | --- |
| TBD | Revised content to align with Aerospace standards, changed document title, added terminology table, refined responsibilities. | Mike Signaigo |