# Procedure Statements

  - Store Items are not tagged with Stock tags and no traceability is maintained.

  - Traceability to Heat and/or Lot number from mill certifications is maintained as required Using Appendix 2 located in the WI 8.1 Product Identification Work Instruction.

  - Any products not identified in the tables are designated as non-color-coded products.

  - Each Branch is responsible for maintaining a clear and consistent method for identifying/marking products with special requirements (e.g. DFAR’s) as needed and segregating Pack# and Heat/Lot# in the same pan.

  - Color coding in the Store/Rems areas may be used but is not controlled nor a requirement of the Product Identification process.

  - Material painted Purple indicates material that has been reclassified as non-traceable.

  - There are some Materials that we do not apply paint to as follows:

    - Aluminum sheet

    - 6063 tubing

    - Stainless sheet

    - Stainless Polished, Satin tubes

    - Copper sheet

    - Brass sheet

    - Cold Rolled sheet

    - Galvanized sheet

    - Corrugated

    - Customer Dedicated

    - Brokered

# Risks and Mitigations

| Risks | Mitigations |
| --- | --- |
| Lack of understanding requirement. | Verify competency and knowledge of associates, training records. |
| Loss of identification therefore loss of traceability which will result in financial loss as material is scrapped or regraded to Rems. | Product Handling, Storage and Preservation (QSP 15.1), In-Process Inspection (QSP 10.2). |
| Color Blindness. | Awareness, QSI 10.1.1, Receiving Inspection (QSP 10.1), In-Process Inspection (QSP 10.2). |
| Inadequate identification materials available. | Overstock, local supply sources, storage locations, supply requests. |
| Potential devaluation of material from paint or glue. | Train and identify what qualifies as cosmetically critical material (see Procedure Statement), knowledge and competency of Product Handling, Storage and Preservation (QSP 15.1). |

Reference and Applicable Documents

QSM-9000	Quality Processes Manual Appendix 1	Material Color Codes

Appendix 2	Material Traceability Requirements

# Personnel to be Trained

Warehouse Personnel Purchasing Personnel Sales Personnel

Branch Admin/Quality Manager

# Revisions Table

| Date | Description of Change | Approved by |
| --- | --- | --- |
| 9/8/21 | Updated document format, added visuals and Appendices. | Melissa DaSilva Penny Brown |