Approvals

| Approved by Quality Manager: | Date: | Approved by Repair Station Manager: | Date: |
| --- | --- | --- | --- |
| Brian R. Olea |  | Sherri Gull |  |

Revision Log

| Revision | Date Revised | Author | Purpose of Revision |
| --- | --- | --- | --- |
| A | 13 Jul 2020 | Brian R. Olea | Initial Release |

List of Effected Pages

| Subject | Page | Date |
| --- | --- | --- |
| Initial Release | all | 13 Jul 2020 |

Contents

Process Acceptance and Scope	3

How will it be used	5

How Remote Conformity Shall Not be Used	5

Failure of Equipment or incomplete inspection	5

Improvement	5

Documented Information	6

Internal Audit Process	6

Inspection for the Issuance of 8130-3.	6

Appendix 1	7

Using Remote Technology for the Purpose of Inspection

### Process Acceptance and Scope

- The purpose of this procedure is to allow for remote conformity inspection and the issuance of 8130-3 by qualified Quality Control Inspectors approved to release articles back into service for the approved MRO.

- This procedure is used in conjunction with SPR-P-852-10 (Issuance of FAA form 8130-3).

- This procedure was created to use remote technology for performing an inspection or issuing a FAA Form 8130-3 by a qualified and approved Quality Control Inspector.

- Burrana has implemented a process and submitted for its approval to the FAA in order to have it approved per this established procedure:

  - That meets the expectations laid out in 14 CFR Parts 145.201 (3), 145.155

  - That meets the requirements laid out in PS-AIR-21-1901

  - That meets the requirements of the Burrana Repair Station Manual

- Following FAA approval of this procedure changes other than those to:

  - grammar,

  - sentence structure,

  - formatting

  - or other changes that do not affect content

done to this procedure shall be submitted to the FAA for approval before those changes are implemented.

References:

PS – AIR – 21 – 1901 Use of Remote Technology During the Performance of Inspections and Inspections

FAA 14 CFR part 145

FAA form 8130-3

SPR-P-852-10 Issuance of FAA form 8130-3

Responsibility

- Burrana takes into consideration the skills and training of the Authorized Quality Control Inspector completing the inspection and the supplier’s employee presenting that remote inspection.

  - It is the responsibility of the Burrana Quality Control Manager to verify the employees listed in the approved list have the appropriate level of competency and that training is up to date. This data is captured within the uniPoint controlled document system or in the Training Roster held at the manufacturing PAH.

  - It is the responsibility of the Burrana Quality Control Inspector to determine the appropriate tools, environment, inspections, and key characteristics are identified prior to the implementation of the process for remote conformity.

  - Burrana will only use Remote Technology for a conformity on an Approved Supplier.

- Proof of competency for the Quality Control Inspector must be demonstrated through

  - Completion of the FAA 8130-3 class

  - Training to the Burrana uniPoint inspection module

  - Completion of Burrana on the job training

  - Training to this procedure and the associated forms

    - 8130-3 ARD (Authorized Release Document)

    - 8100-1 Inspection Record

    - SPR-F-852-50 Remote Conformity Checklist

and documented in the employee files on the QMS software uniPoint.

- Tools and environmental controls

  - Tools used for remote inspection shall be calibrated and the tool number and last calibration date recorded on the Inspection form.

  - Video resolution shall not be less than 720p unless there is evidence that the resolution used supports the inspection requirements.

  - Audio equipment in the form of a headset to block out background noise will be used.

  - The environment used to do the remote conformity shall be free of excess distractions and controlled to the degree necessary to give confidence in the remote technology being used for the conformity.

  - When necessary, multiple angles will be required to review key characteristics as well as extra lighting. When this occurs, those will be requested and noted on the SPR-F-910-50 form as listed in the Documented Information section of this document.

  - There are no unique inspection devices needed for the conformity of Burrana IFE Products.

  - Burrana will record, at random, Remote Conformities in order to include a review in the internal audit.

- In the event of an interruption

  - Burrana will re-validate any inspection that is interrupted by environmental or mechanical failures in order to maintain confidence in the inspection.

References:

PS – AIR – 21 – 1901 Use of Remote Technology During the Performance of Inspections and Inspections

CORP-P-720-01 Competence & Awareness

CORP-P-720-02 Certificate Training

CORP-P-715-01 Control of Monitoring and Measuring Equipment

CORP-P-870-02 COS Response Team

F-720-01 Training Action Plan

F-720-02 Training Roster

F-720-12 On the Job Training (OJT)

8130-3 certificate training course completion certificate

SPR-F-910-50 Remote Conformity Checklist

### How will it be used

- Burrana will use Remote Technology in the form of live Audio and Video feeds to conform Repaired parts from our approved supplier location. These Remote Inspections shall never:

  - be pre-recorded

  - be conducted without scheduled timeframes

  - use equipment not suitable to the inspection

  - utilize a resolution without enough detail to approve, without doubt the parts conform to the written requirements.

- Burrana IFE (In Flight Entertainment) is considered a Category 2 part in the CPL reference document, and therefor remote technology would be considered appropriate when properly controlled.

If at any time the status changes to a Category 1 part, this process will be re-evaluated and submitted to the FAA for additional approvals.

References:

PS – AIR – 21 – 1901 Use of Remote Technology During the Performance of Inspections and Inspections

CORP-P-715-01 Control of Monitoring and Measuring Equipment

Burrana Repair Station Manual

CPL Summary

### How Remote Conformity Shall Not be Used

- Burrana will take into consideration the complexity, Novelty, and Safety Criticality of the article to be inspected remotely. If, for any reason, the article to be inspected should fall into a category 1 on the FAA CPL it would be excluded from remote conformity consideration.

### Failure of Equipment or incomplete inspection

- If for any reason, there is an equipment failure:

  - A secondary inspection will be conducted if the failure can be rectified or

  - The product will be shipped to the Burrana part 145 approval holder location to complete the inspection per Burrana’s requirements or

  - A qualified quality control technician will travel to the facility to inspect in person.

References:

PS – AIR – 21 – 1901 Use of Remote Technology During the Performance of Inspections and Inspections

Paragraph 13, part 2

### Improvement

- The Burrana Quality Control Inspector will document in the uniPoint Corrective Action module using the Continual Improvement task, any failures or opportunities for improvement discovered during the remote viewing process.

  - This is to allow for future improvements and review by the management team to create a more secure and reliable system for remote inspection.

References:

CORP-P-1010-01 Improvement

CORP-P-1020-01 Nonconformity Corrective Action

CORP-P-870-01 Control of Nonconforming Outputs

### Documented Information

- The Burrana Quality Control Inspector completing the remote inspection will document the inspection process using the

Documents reviewed and Inspections used to determine conformity during the remote inspection will be recorded on the inspection checklist SPR-F-910-50.

- All records associated with the Remote Inspection shall be considered documented information and retained for at least 10 years following the end of a project.

References:

CORP-P-750-01 Document Control

FAA Form 8100-1 Current Revision

FAA Form 8130-3 Current Revision

SPR-F-910-50 Remote Conformity Checklist

8130-3 training completion certificate

### Internal Audit Process

- Burrana follows an internal audit process, which is documented in CORP-P-920-01 (Internal Audits) and recorded in uniPoint for retrieval and review. The process to review the remote conformity technology procedure will be included in this audit cycle as a separately identified procedural audit and will include:

  - Review of this procedure

  - Review of supporting documents used during remote inspections

  - Review of 3 (if available) remote conformity approvals, at random, to verify process adherence

  - Review of training records and certificates for those approved to complete remote conformities

References:

CORP-P-920-01 Internal Audits

CORP-P-930-01 Management Review

### Inspection for the Issuance of 8130-3.

- Burrana will follow the Process Flow Diagram (appendix 1) for determination of remote conformity needs.

This document will also be required to be reflected in the suppliers Quality Documentation regarding Inspections.

References:

CORP-P-851-01 Control of Production and Service Provision

CORP-P-811-01 Operational RISK Management

### Appendix 1