# Objective

This SOP defines the inspection procedure of PMA and prototype parts.

Note: Inspection of PMA and prototype parts by vendors is outside the scope of this document. They are required to follow “A-QS-100-01 Supplier Quality Requirements (SQR) PMA”.

# Responsibilities

Quality Inspection is responsible to follow and maintain this procedure.

## Definitions & Acronyms

| - Term | - Meaning |
| --- | --- |
| - ATP | - Acceptance Test Plan |
| - COTS | - Commercial Off-The-Shelf |
| - FAI | - First Article Inspection |
| - FAIR | - First Article Inspection Report |
| - NCR | - Nonconformance Report |
| - PMA | - For purposes of this document, PMA are parts that the Company has FAA-PMA |
| - PMA Inspector | - An individual selected to perform First Article and entry into service inspections of PMA articles. The Quality Manager selects the PMA Inspector based on experience, training, and by demonstration of ability |
| - PO | - Purchase Order |
| - Prototype | - Of any mechanical device, a production model suitable for complete evaluation of mechanical and electrical form, design, and performance |

# Inspection Process

## FAI (First Article Inspection)

Each Company part number is subject to first article inspection, independent of similar parts or multiple parts on a single drawing. A FAIR is to be completed by Inspection for any of the following conditions:

- The PO requires the vendor to provide a FAIR

- First time buy of any PMA Prototype parts or first time PMA Prototype parts made to a print revision

- First time part bought from the vendor (excluding COTS)

- When a FAIR is provided by the vendor due to a change

- More than 2 years since last received from the providing vendor (excluding COTS)

- When directed by Inspection Manager or Engineering

A FAIR is to include all details from sub-components of an assembly, or the sub-components are to have separate FAIRs with the exception of COTS items. The same applies to vendor protect part numbers to the final Burrana part number.

## Inspection Damage

Parts damaged in inspection are to be removed from inventory and given to the Project Engineer for use in the engineering parts library or scrapped in accordance with the procedure.

## Measurement Considerations

When Burrana does not have the ability to measure or verify all part features. For example - features only accessible before assembly, features that require specialty measurement equipment, parts that must remain in air-tight packaging, and destructive testing. In these cases, Burrana will use supplier certification for these features, request third party verification of these features, or analyze a sample part in a manner that renders the sample unusable.

# Governing Documents

- 14 CFR 21.137(e) thru (k)

- AS9100

- AS9102

# Forms and Documented Information

## Document Control

FAIR - First Article Inspection Report (AS 9102). All approved FAIRs are to be stored electronically on the server.

Inspection results are filed in the uniPoint database, the ERP, dropbox, SharePoint, or hard copies.