# Purpose & Scope

To describe the process for the receiving inspection of incoming material.

# Reference Documents

QSP 8.1	Product Identification QSI 10.1.1	Material Measurements

QSP 13.1	Control of Nonconforming Product

QSP 15.1	Product Handling, Storage and Preservation

# Process Flow

Is the item an MTOS-Material to Outside Shop PO?

No

A

Yes

Matches?

No

B

Yes

C

Notifies Receiving Okay to proceed.

Buyer verifies Packing List reflects what should be received, including quantities, sizes, etc.

Forwards copy of Receiving Paperwork to the Buyer or Backup.

Obtains the paperwork for the delivery. Verify packing slip or stock transfer against Business System to verify correct delivery location.

Receiving	3.1

Receiving	3.2

Receiving	3.3

Buyer	3.4

Buyer	3.5

Buyer	3.6

A

Receiving	3.7

Receiving	3.8

Receiving	3.9

Receiving	3.10

Note: At this point, we have not unloaded or checked material physically

Is material a Stock Transfer?

Continue checking shipping paperwork vs. Business System PO to verify additional information including material alloy, grades, size, quantity and receipt of needed Test Reports.

No

Matches?

Notifies Buyer of discrepancy.

No

B

Yes

D

Yes

C

Buyer	3.11

Buyer	3.12

Receiving	3.13

Can shipment be refused?

Refer to Procedure Statement 4.6.

No

Yes

Notify Receiving to refuse shipment.

Refuse shipment, noting reason on packing slip, signing it and returning original to Supplier and forward copy to scanning and copy to Buyer.

E

E

Buyer	3.14

Buyer	3.15

Modifies PO in the Business System and re-print. See Note 4.7.

Notifies Receiving to proceed.

C

Receiving	3.16

Per visual inspection

by Receiving, is	No

material acceptable to unload?

Receiving	3.17

Yes

No

Evaluate per 4.6, can we refuse shipment?

Yes

Buyer	3.18

Receiving	3.19

Receiving	3.20

Is material FOB the Mill?

(E.g. Nucor)

Refuse shipment, noting reason on Packing Slip, returning original to Supplier and send copy of paperwork to scanning and a copy to Buyer.

No

Yes

F

Packing slip

Stamp original packing slip, “Subject to

inspection weight and piece count.” stamp (see Figure 1) Sign, date in stamp area, and return original to Supplier. Retain duplicate packing slip.

D

D

Unload order from truck and place in hold area. Store material, forward PO (print copy if necessary), Packing Slip and Product to designated Store Receiving Hold Area(s).

Receiving	3.21

|  | Inspect material per packing slip or stock transfer for correct material, alloy, special requirements, weight, quantity and quality per QSI 10.1.1. |  |
| --- | --- | --- |

Receiving,	3.22 Store Receiving

Receiving,	3.23 Store Receiving

Receiving,	3.24 Store Receiving

Material Acceptable?

Yes

Material for immediate release with order?

No

Color code material, and place material in a broker item rack or in a staging location.

|  | Process any non-conformances per QSP 13.1. |  |
| --- | --- | --- |

Yes

Pull paperwork from “Hold for Incoming” file or print copy from the system, note location of material, quantity (piece count, length or weight of material—must match quantity received), Pack #,

P.O. # and Heat/Lot # next to line item on order and sign next to “Puller.” Attach copies of test report to paperwork and forward to Will Call/Shipping Office.

No

Receiving,	3.25 Store Receiving

Receiving	3.26

Receiving,	3.27 Store Receiving

Is MTOS PO?

Post receipt of material in Business System.

Notify Buyer to proceed.

No

Yes

G

G

Stock Tag

Printing needed Stock Tags per QSP

8.1 unless material is being immediately released to Will Call or Shipping department for an order.

Moves received tagged stock material out of hold area to primary stock location or overstock.

Review that all test reports match received material and IMS purchase order requirements, affix pack labels to test reports and forward all original test reports to Scanner.

Receiving,	3.28 Store Receiving

|  | Identify and color code material to be stocked per QSP 8.1 |  |
| --- | --- | --- |

Receiving,	3.29 Store Receiving

| Applies Inbound Inspection stamp (see Figure 2) to packing slip(s) and record required information and initials. Forward to Accounts payable department or Branch designee. |  |  |
| --- | --- | --- |
|  |  | Packing Slip |

Receiving,	3.30 Store Receiving

Receiving or	3.31 Designated Employee

Stocker,	3.32

Store Receiving

F

Immediately after receiving the inventory, enter the Supplier Return. This must be done immediately so that inventory does not appear as available for sales.

Receive the material into the business system.

Forward correspondence to the Trucking Company, this could include copies of the Mill Invoices for Material, Pictures, Packing Slip, and any other necessary information.

Contact the Trucking Company to notify them of the rejection. If possible, get an email address to communicate going forward, so all communication is recorded.

If the rejected material still needs to be ordered for inventory, give the Supplier a new PO for the replacement order.

Contact the material Supplier to notify them of the issue and get the Contact information for the Trucking Company.

Forward an additional copy of the Packing Slip and Photos of the load to the Buyer.

Refuse shipment noting reason on Packing Slip, return original to the carrier, and forward a copy to scanning.

Receiving	3.33

Receiving	3.34

Buyer	3.35

Buyer	3.36

Buyer	3.37

Buyer	3.38

Buyer	3.39

Buyer	3.40

# Procedure Statements

  - Responsibility

    - The functions assigned to “Receiving” may be performed by the following personnel: Receiving Supervisor, Receiving Administrator, Receiving Inspector, or anyone identified by management as trained and qualified personnel.

    - The functions assigned to “Buyer” may be performed by the following personnel: Product Specialist, Branch Buyer, Broker Buyer, or anyone identified by Management trained in Purchasing functions.

    - All Stock Transfers are received regardless of condition or accuracy. Refer to work instructions for Stock Transfer Resolution as needed.

# Verification of Material/Test Certifications

    - Mill Test Report – For the purposes of IMS’ business the Mill Test Report, Test Report or MTR, as referenced in this and all other QSPs, QSIs and other communication & documentation, is a general designation for the document that is provided by the Supplier that identifies the product and type of material, including any required specifications, test results, etc. Acceptable document names include, but are not limited to, Mill Certificate, Certified Test Report, Test Report, Quality Inspection Report, Inspection Certificate, etc.

    - Heat Number – For the purposes of IMS’ business the Heat# or Heat/Lot#, as referenced in this and all other QSPs, QSIs and other communication & documentation, is a general designation that refers to the Unique Identification on the MTR that links the MTR to the material. If material is Line Marked that is the Identification #, if the material is not Line Marked the # on the MTR that can be used as the Unique Identifier is used. Examples include, but are not limited to, Heat#, Lot#, Coil ID, Metal ID, Cast# and other variations.

    - Heat Numbers are recorded into the IMS Business System and on the Stock Tags using only Alpha and Numeric characters, any other characters including but not limited to, dashes (-), slashes (/) and spaces, are disregarded. Example: 1Q-6B/52 00 would be recorded as 1Q6B5200.

    - If the Purchase Order requires material test reports, or other certifications, the Receiving Inspector verifies that the correct certifications are received. Verify correct size, grade, specification, and Heat number matches the material. Verification of this is denoted by initialing the quality stamp. The individual that is completing the quality inspection is professionally declaring the product is correct.

# 4.0	Procedure Statements

    - All original MTR’s are labeled and forwarded to the scanner, even if a copy was attached to an order for immediate release.

    - If the MTR’s are unacceptable, or not received as required, then the Receiving Inspector or Purchasing contacts the Supplier and the material is placed on hold. Material is tagged with an On Hold tag HT001.

      - If the MTR’s are corrected and received within two working days, then the Receiving Inspector attaches packing slip to the MTR and forwards them to the Receiving Administrator.

      - If the MTR’s are not received/corrected within two working days, then a Rejection Notice, RN001, is initiated in accordance with QSP 13.1.

# “Hold for Incoming” Orders

    - Branches are responsible for defining a system that alerts the Receiving Dept. that an inbound P.O. is for a pending order. This can include, but is not limited to “Hold for Incoming” files by Date, or by Supplier, or notes on the footer of the P.O., brokered part numbers, etc.

    - When orders print, which are on hold for receipt of broker orders or other incoming material, the paperwork is forwarded to the Receiving or Shipping Office and the Receiving or Shipping Department files by date expected.

      - When incoming material is received, the paperwork is forwarded to the Shipping Office per the process in section 3.25.

      - Receiving reviews all orders still in “Hold for Incoming” for that date. Notify Buyer for P.O. referenced on paperwork that material has not arrived.

      - Each branch needs to specify someone to manage “Hold for Incoming” so that customer orders are on time and customer is notified if material will be late.

# Second Shift Receiving Inspection

Receiving inspection conducted by the second shift is similar to that done by the first shift except that it is done in a different sequence.

# 4.0	Procedure Statements

    - The material is picked up from Suppliers by the Industrial Metal Supply drivers and the packing slips are placed in slots according to the truck number in the Shipping Office.

    - The Receiving Inspector collects the packing slips from the Shipping Office.

    - Material is first unloaded and inspected per the packing slip, then matched to the PO.

    - Material found to be non-conforming is put in the hold rack and tagged with an On Hold Tag (HT001) for the Receiving Supervisor to evaluate and document as required the next day.

# Receiving Appointments

Appointments to deliver material should be recorded in the network Receiving Appointments Excel file if necessary

# Rejection Process

    - Truck is in Receiving and the Receiving Dept. has identified there is an issue with the load.

    - Receiving Dept. contacts the Branch Decision Maker to determine if the load is rejectable.

    - If determined material it is rejectable, the Receiving Dept. takes pictures immediately of the damaged material. (Take pictures of material when material is Damaged, Wet, Rusty, etc. No need to take pictures if material is simply the wrong size.)

    - Receiving Dept. completes the Rejection Notice and emails a copy with the pictures to the Buyer/Product Specialist that issued the Purchase Order and copy’s the Supply Chain Mgr.

    - The Receiving Dept. follows-up the email by calling the Buyer/Product Specialist for immediate action since the driver is waiting.

    - If the Buyer/Product Specialist cannot be reached then the Receiving Dept. calls the Branch Buyer or Backup Person and forwards them a copy of the email w/ pictures to handle the rejection.

    - The Product Specialists or Branch Buyer will take action by contacting the Supplier to notify them of the rejection and work out resolution as to whether the truck will be rejected or if material will be unloaded and picked-up at another time.

# 4.0	Procedure Statements

    - If someone other than the original buyer of the PO handles the rejection, they should forward any correspondence to bring the originator into the loop.

# Modifying P.O.

    - When incorrect material is accepted and not agreed to keep, receive the material to P.O. and process Supplier Return after receipt per QSP 13.1

    - When incorrect material is accepted and agreed to keep, add a new line with that part number to the P.O. so it can be received.

# Risks

  - Adequacy of receiving requirements communicated through supply chain:

    - Clarity of requirements, clear understanding, unambiguous definitions.

    - Approval criteria well defined for products, services, special processes, equipment, acceptance, buy-off, delegations, permissions and authorizations.

    - Competency requirements and qualifications (skills, training, evaluation).

    - Control of monitoring equipment for inspection and verification (requirements, instructions, acceptance criteria, and properly maintained equipment).

  - Arrangements to control equipment, and tools including:

    - Validating the equipment, and tools with test pieces and charts.

    - Maintaining equipment, tools for proper configuration, change authorization, planned maintenance, and calibration.

    - Proper storage of equipment and tooling for correct identification.

    - Proper environment conditions (space, cleanliness, temperature, humidity).

    - Proper preservation and protection (corrosion, deterioration, loss prevention).

    - Proper use of asset registers, condition checks, segregation (including equipment/tooling belonging to others).

| 6.0 | Applicable Records |  |
| --- | --- | --- |
|  | Packing Slip |  |
| 7.0 | Personnel to be Trained |  |
|  | Receiving Personnel Purchasing Personnel Stocker Branch General Manager Store Manager Inside Sales Manager Warehouse Manager |  |
|  | Figure 1 | Figure 2 |