# Purpose & Scope

To establish a procedure for the in-process inspection of cut material.

# Reference Documents

QSP 8.1	Product Identification

# Process Flow

No

Acceptable?

Yes

Acceptable measurement?

No

Yes

Record nonconforming product on ID001 form and put product in designated area (e.g. scrap, rems, etc.)

ID001

A

A

Set up a piece, bundle or stack of material on machine and make trim cut if necessary.

Make first cut and verify cut length per Work Order and QSI 10.1.1.

Adjusts machine setup and inspects cut pieces since last acceptable piece.

Periodically inspect cuts to insure they are within tolerance.

Pull appropriate material or verifies material pulled by others for order matches order requirements by confirming grade, alloy, size, quantity and stencil (if available).

Adjust machine. Record nonconforming product on ID001 form and put product in designated area (e.g. scrap, rems, etc.)

ID001

Machine/Saw	3.1

Operator

Machine/Saw	3.2

Operator

Machine/Saw	3.3

Operator

Machine/Saw	3.4

Operator

Machine/Saw	3.5

Operator

Machine/Saw	3.6

Operator

A

Verify total quantity cut and sign the Work Order as Operator.

If necessary, setup more material and repeat steps 3.2 through 3.6 for each setup.

Machine/Saw	3.7

Operator

Machine/Saw	3.8

Operator

# Procedure Statements

  - The Machine/Saw Operator verifies the alloy, grade, dimension and condition of material against Work Order and records Pack number, Lot/Heat Number and verifies number of pieces on Work Order.

  - While producing parts to the requirements of a Work Order, place a check mark next to each requirement or comment to ensure requirement is met.

  - As a general practice, Operator will dimensionally inspect first, middle, and last parts. When bundle cutting, randomly measure pieces from top, middle and bottom of cut.

  - When checking that parts meet requirements use the operator notes area to record any notes.

  - Operator certification is sufficient to show requirements compliance.

  - Statistically monitor compliance by categorizing and monitoring Credit Notes or complaints.

# Risks

- Establishment of criteria for acceptance of products (measurement, monitoring, inspection, verification, and validation).

- Inspection and verification using the correct specs, acceptance criteria, and equipment.

- Maintaining proper records to provide evidence that product qualification meets all defined requirements e.g. inspection history, test reports, validation reports, and CofC.

- Control of product attributes e.g. drawings, specifications, standards, production instructions, re-work.

- Verification of changes to customer orders e.g. change notices, amendments, etc.

- Use of special identification methods to show nonconforming status (segregation, tags, labels and markings).

- Using an out-of-tolerance tool or instrument.

- Spurious readings on equipment (imprecision).

# Applicable Records

Inventory Disposition Form  ID001

# Personnel to be Trained

Warehouse Manager Shift Supervisor Warehouse Lead Machine/Saw Operator