# Purpose and Scope

This process describes the procedure for identifying product by suitable means from receipt through delivery and for recording unique identification including Pack number, Lot number and/or Heat number.

# Process

- Receiving Inspector

Identify material with Stock tags. A minimum of one Stock tag is required on each bundle or skid.

- Receiving Inspector

Paint at least one end of material as required per Appendix 1 Tables 1 through 8.

See Procedure Statement B.

# Process

- Receiving Inspector

If tagging is not appropriate (e.g. large rounds) mark Pack# and Heat/Lot# on each piece.

- Receiving Inspector

For skids of sheet or plate, attach colored paper with alloy type identified as required per Appendix 1 Table 1.

- Material Handler

Maintain/transfer identification, including Pack# and Heat# or Lot# on unmarked material that is being returned to stock for future use, as necessary and per steps

1-4 above.

# Procedure Statements

  - Store Items are not tagged with Stock tags and no traceability is maintained.

  - Materials are painted in accordance with Appendix 1 Tables 1 through 8 to indicate material type and thickness. If material is surface or cosmetically critical, then paint can be applied to Stock tag or outside packaging and not directly to product.

  - Traceability to Heat and/or Lot number from mill certifications is maintained as required per Appendix 2.

  - Any products not identified in the tables are designated as non-color coded products.

  - The functions may be performed by anyone as identified by Management and trained in the process.

  - Each Branch is responsible for maintaining a clear and consistent method for identifying/marking product with special requirements (e.g. DFAR’s) as needed and segregating Pack# and Heat/Lot# in the same pan.

  - If material does not have stenciling or is deemed as a fairly expensive product that potentially runs the risk of losing traceability because of only having one tag, then the Branch may need to tag bundles on both ends.

  - Colors used in this document are examples only. Paint and paper colors may not exactly match.

  - Color coding in the Store/Rems areas may be used but is not controlled nor a requirement of the Product Identification process.

  - Material painted Purple indicates material that has been reclassified as non-traceable.

  - Material that we do not apply paint to is:

    - Aluminum sheet

    - 6063 tubing

    - Stainless sheet

    - Stainless Polished, Satin tubes

    - Copper sheet

    - Brass sheet

    - Cold Rolled sheet

    - Galvanized sheet

    - Corrugated

    - Customer Dedicated

    - Brokered

# Risks and Mitigations

| Risks | Mitigations |
| --- | --- |
| Lack of understanding requirement. | Verify competency and knowledge of associates, training records. |
| Loss of identification therefore loss of traceability which will result in financial loss as material is scrapped or regraded to Rems. | Product Handling, Storage and Preservation (QSP 15.1), In-Process Inspection (QSP 10.2). |
| Color Blindness. | Awareness, QSI 10.1.1, Receiving Inspection (QSP 10.1), In-Process Inspection (QSP 10.2). |
| Inadequate identification materials available. | Overstock, local supply sources, storage locations, supply requests. |
| Potential devaluation of material from paint or glue. | Train and identify what qualifies as cosmetically critical material (see Procedure Statement), knowledge and competency of Product Handling, Storage and Preservation (QSP 15.1). |

Reference and Applicable Documents

QSM-9000	Quality Processes Manual Appendix 1	Material Color Codes

Appendix 2	Material Traceability Requirements

# Personnel to be Trained

Warehouse Personnel Purchasing Personnel Sales Personnel

Branch Admin/Quality Manager

# Revisions Table

| Date | Description of Change | Approved by |
| --- | --- | --- |
| 9/8/21 | Updated document format, added visuals and Appendices. | Melissa DaSilva Penny Brown |

Table 1 – Sheet and Plate Skids

| Material | Paper/Paint Color |  | Recorded Information |
| --- | --- | --- | --- |
| Copper | Pink |  | Thickness (or Wgt/Sqft) x Width x Length |
| Brass | Yellow |  | Thickness x Width x Length |
| 2024 Aluminum | Brown |  | Thickness x Width x Length |
| 3003 Aluminum | Pink |  | Thickness x Width x Length |
| 4017 Aluminum | Red |  | Thickness x Width x Length |
| 5052 Aluminum | Green |  | Thickness x Width x Length |
| 5086 Aluminum | Orange |  | Thickness x Width x Length |
| 6061-T4 Aluminum | White |  | Thickness x Width x Length |
| 6061-T6 Aluminum (Import) | Gold |  | Thickness x Width x Length |
| 6061-T6 Aluminum (Domestic) | Blue |  | Thickness x Width x Length |
| 6061-T6 Aluminum (DFARs) | White | Blue | Thickness x Width x Length |
| 6061-O Aluminum | Yellow | Green | Thickness x Width x Length |
| 7075 Aluminum | Pink | Blue | Thickness x Width x Length |
| Aluminum Tooling | White |  | Thickness x Width x Length |
| Cold Rolled | Pink |  | Gauge (or Thickness) x Width x Length |
| Galvanized G90 | White |  | Gauge (or Thickness) x Width x Length |
| Galvanized G60 | Brown |  | Gauge (or Thickness) x Width x Length |
| Galvanized G40 | Green | Brown | Gauge (or Thickness) x Width x Length |
| Galvannealed (Jet Coat) | Blue |  | Gauge (or Thickness) x Width x Length |
| Electro-Galv (Paintlok) | Yellow |  | Gauge (or Thickness) x Width x Length |
| Bonderized | Orange |  | Gauge (or Thickness) x Width x Length |
| Hot Rolled Pickled & Oiled | Yellow + Table 3 |  | Gauge (or Thickness) x Width x Length |
| Hot Rolled | No Color + Table 3 |  | Gauge (or Thickness) x Width x Length |
| Hot Rolled GR50 | Pink + Table 3 |  | Gauge (or Thickness x Width x Length |
| Hot Rolled A606 (HSLA/Corten) | Blue | Red | Gauge (or Thickness) x Width x Length |
| AR400 | Blue |  | Gauge (or Thickness) x Width x Length |
| AR500 | Orange |  | Gauge (or Thickness) x Width x Length |
| 4130 | Blue | Gold | Thickness x Width x Length |
| Corrugated Galvanized | White |  | Gauge (or Thickness) x Width x Length |
| Corrugated Galvalume | Orange |  | Gauge (or Thickness) x Width x Length |
| Corrugated (no coating) | Pink |  | Gauge (or Thickness) x Width x Length |
| Stainless 304 2B or MF | Red |  | Gauge (or Thickness) x Width x Length |
| Stainless 304 #4 | Blue |  | Gauge (or Thickness) x Width x Length |
| Stainless 304 #8 | Yellow |  | Gauge (or Thickness) x Width x Length |
| Stainless 316 2B or MF | Green |  | Gauge (or Thickness) x Width x Length |
| Stainless 316 #4 | Green | Blue | Gauge (or Thickness) x Width x Length |
| Stainless 430 2B or MF | Orange |  | Gauge (or Thickness) x Width x Length |
| Stainless 430 #4 | Orange | Blue | Gauge (or Thickness) x Width x Length |
| Stainless 430 BA | Orange | Pink | Gauge (or Thickness) x Width x Length |

Paper tags or paint can be used to identify material.

If colored paper is not available then paper tag, stock tag, skid, or material can be painted with applicable color designation.

If no color designation is listed, White tags can be used.

Table 2 – Long Products (Bars, Shapes, Tube & Pipe)

| Material/Alloy | Paint Color |
| --- | --- |
| Stainless Type 303** | Pink |
| Stainless Type 304** | Yellow |
| Stainless Type 316** | Blue |
| Stainless Type 17-4 | Green |
| Cold Rolled 1018 | Red |
| Cold Rolled 12L14 | Gold |
| Cold Rolled 1045 | Black |
| 4140/4142 Alloy | Green |
| 4130 Alloy | Pink |
| DOM | Gold |
| Hot Rolled | No Color + See Table 3 |
| Rebar | No Color + See Table 3 |
| Hot Rolled Pickled & Oiled | Orange |
| Hot Rolled Standard Beam | Blue |
| Hot Rolled W-Beam | Yellow |
| Uncoated Pipe | Silver |
| Coated (Black) Pipe | No Color |
| Aluminum 6061 | Blue |
| Aluminum 6101 | Green |
| Aluminum 7075 | Red |
| Aluminum 5086 | Orange |
| Brass* | Red |
| Copper* | Orange |

*Paint should be applied to Stock tag or outside packaging.

**Stainless True Bar add Black to designated colors to identify as True Bar.

Table 3 – Hot Rolled Steel Sheet, Plate, Tube, Angle, Tee, RD Bar, SQ Bar, Flat Bar, and Rebar

| Thickness | Paint Color | Thickness | Paint Color |
| --- | --- | --- | --- |
| 18 Gauge | Pink | 1/2” | White |
| 16 Gauge | White | 5/8” | Green |
| 14 Gauge | Green | 3/4” | Black |
| 13 Gauge | Red | 7/8” | Silver |
| 12 Gauge | Gold | 1” | Pink |
| 11 Gauge | Orange | 1-1/8” | Orange |
| 10 Gauge | Pink | 1-1/4” | Yellow |
| 7 Gauge | Black | 1-1/2” | White |
| 1/8” | Orange | 1-3/4” | Black |
| 3/16” | Blue | 2” | Pink |
| 1/4” | Yellow | 2-1/2” | White |
| 5/16” | Green | 2-3/4” | Black |
| 3/8” | Red | 3” | Pink |

For Hot Rolled Picked & Oiled Sheet/Plate items add Yellow to designated colors to identify as P/O material. For Laser Quality Sheet/Plate items add Green to designated colors to identify as LQ material.

Table 4 – Flat Expanded Metal

| Size | Paint Color |
| --- | --- |
| #9 | Orange |
| #13 | Red |
| #16 | White |
| #24 | Yellow |

For Raised items add Blue to designated colors to identify as Raised vs. Flat.

For Galv Expanded Sheet add Black to designated colors.

Table 5 – Steel Channel

| Size | Paint Color | Size | Paint Color |
| --- | --- | --- | --- |
| 3” x 4.1 lbs/ft | Blue | 6” x 8.2 lbs/ft | Red |
| 3” x 3.5 lbs/ft | Red | 6” x 10.5 lbs/ft | Pink |
| 3” x 5.0 lbs/ft | White | 6” x 12 lbs/ft | Brown |
| 4” x 4.5 lbs/ft | Brown | 7” x 9.8 lbs/ft | Yellow |
| 4” x 5.4 lbs/ft | Gold | 8” x 11.5 lbs/ft | Orange |
| 4” x 6.25 lbs/ft | Yellow | 8” x 13.75 lbs/ft | Red |
| 4” x 7.25 lbs/ft | Green | 9” x 13.4 lbs/ft | Green |
| 4” x 13.8 lbs/ft | White | 10” x 15.3 lbs/ft | Blue |
| 5” x 6.7 lbs/ft | Silver | 10” x 20.0 lbs/ft | White |
| 5” x 9.0 lbs/ft | Black | 12” x 20.7 lbs/ft | Black |

Table 6 – Steel Beams

| I-Beam | Paint Color | WF-Beam | Paint Color |
| --- | --- | --- | --- |
| 3 x 5.7# | Blue | 4 x 13# | Blue |
| 3 x 7.5# | Green | 5 x 16# | Green |
| 4 x 7.7# | Red | 5 x 19# | Red |
| 4 x 9.5# | Orange | 6 x 9# | Orange |
| 5 x 10# | White | 6 x 12# | White |
| 5 x 14.75# | Yellow | 6 x 15# | Yellow |
| 6 x 12.5# | Brown | 6 x 16# | Brown |
| 6 x 17.25# | Blue | 6 x 20# | Blue |

Table 7 – Steel Pipe

| Size | Paint Color |
| --- | --- |
| Schedule 10 | Blue |
| Schedule 40 | No Color |
| Schedule 80 | Yellow |

For Uncoated Pipe add Silver to the designated colors to identify difference between Uncoated vs. Coated.

Table 8 – Specialty Items

| Item Designation | Paint Color |
| --- | --- |
| Brokered | Fluorescent Orange |
| Customer Dedicated | Fluorescent Green |

Paint should be applied to Stock tag or outside packaging for Brokered and Customer Dedicated product.

| Mill Certification Traceability maintained by Heat# or Lot# | Certificate of Conformance Traceability maintained by Heat# or Lot# | No Certification Traceability not maintained |
| --- | --- | --- |
| Aluminum |  |  |
| Shape: Sheet and Plate Alloy: All unless otherwise stated | - Cast Tooling Plate | - Perforated Sheet - Expanded Sheet - Painted Sheet |
| Shape: Long Products Alloy: All unless otherwise stated | Shape: Long Products Alloy: 6063 | - Anodized Tube |
| Stainless Steel |  |  |
| Shape: Sheet and Plate Alloy: All unless otherwise stated |  | - Perforated Sheet - Expanded Sheet |
| Shape: Long Products Alloy: All unless otherwise stated |  |  |
| Carbon Steel |  |  |
| Shape: Sheet and Plate Alloy: All unless otherwise stated |  | - Perforated Sheet - Expanded Sheet - Corrugated Sheet |
| Shape: Long Products Alloy: All unless otherwise stated |  | - HRP/O Strip Bar - Slotted Angle - Chrome Steel Tube - Telescoping Tube - Hammered Tube |
| Copper |  |  |
|  | Shape: All Alloy: All unless otherwise stated | - Foil |
| Brass |  |  |
|  | Shape: All Alloy: All unless otherwise stated | - Foil - Angle - Channel - Tube |
| Miscellaneous |  |  |
|  |  | Surplus and Pre-Cuts |
|  |  | Retail Items |
|  |  | Any Items not identified elsewhere |

- Material Traceability documents must be obtained from Suppliers.