How to use (delete this box after first use if desired)

Copy the table below into Excel or Word. Add or adjust rows to suit new products or customer-specific requirements.
Use the Preservation Level column to match the steps in WI-854-001 Preservation & Packaging Methodology.

| Product / Material Category | Typical Examples | Preservation Level | Packaging Materials | Environmental Controls | Additional Notes |
| --- | --- | --- | --- | --- | --- |
| Finished Electronic Assemblies | PCBA, IFEs, power-supply modules | Level 2 | Static-dissipative moisture-barrier bag + desiccant + outer corrugated carton with foam inserts | ESD handling; RH < 60 % during pack | Add humidity indicator card if export > 30 days |
| Machined Aluminum Parts (Un-coated) | Brackets, housings | Level 2 | VCI* bag + bubble wrap + triple-wall carton | Store < 60 % RH; temp 15 – 30 °C | Apply light rust-inhibitor spray if storage > 6 months |
| Plated / Painted Metal Parts | Chem-film, anodized, powder-coat parts | Level 1 | Poly sleeve + labeled tote for in-house move | No special control if < 24 h | Avoid stacking; separate with foam sheets |
| Composite Structures | Carbon-fiber panels, glass-fiber ducts | Level 3 | Vacuum-sealed barrier bag + desiccant + custom foam-lined crate | Temp 15 – 30 °C; RH < 50 % | Shock recorder for export shipments |
| Electro-mechanical Sub-assemblies | Actuators, fans, motors | Level 2 | Clamshell ESD tray + foam-lined carton | ESD handling | Secure moving parts with transit screws |
| Hazardous Goods (Lithium-ion batteries) | Cabin battery packs | Level 3 | UN-approved haz-mat crate + thermal liner | Follow ICAO/IATA PI 965-II limits | Attach haz-mat labels & SDS; temp < 25 °C |
| Repair / Warranty Returns | Returned units awaiting evaluation | Level 2 | Original packaging if usable; otherwise Level 2 materials | Segregate in “Quarantine” cage | Identify with NCR tag; follow WI-870-001 |
| Spare Parts (Small hardware) | Screws, nuts, O-rings | Level 1 | Zip-lock poly bag + labeled bin/tote | — | Batch labels must show lot & expiry date |
| Long-Term Storage (> 1 year) | Strategic spares | Level 3 | Vacuum-sealed barrier + desiccant + nitrogen-purge crate | Maintain RH < 40 %; annual desiccant change | Review condition every 12 months |

*VCI = Vapor-Corrosion-Inhibitor bag