# Objective

Explain, in detail, how operators identify required environmental limits, record actual readings, respond to out-of-limit events, and file evidence so auditors see full compliance with AS 9100 D 7.1.4.

# Applicability

Covers every production, inspection, test, and storage area where the drawing, specification, or customer contract defines temperature, humidity, ESD, cleanliness, noise, or lighting limits.

# Responsibilities

Process Owner lists required limits on the Environmental Parameter Card posted at the point of use.
Operator / Inspector takes and records readings at the defined frequency, halts work when limits are exceeded, and begins hold/evaluation.
Quality Technician reviews logs daily, trends data monthly, and raises Corrective Action if chronic drift is detected.
Maintenance / Facilities calibrates sensors, changes desiccant, and services HVAC; certificates are filed with calibration records.
Quality Manager summarises environmental status in each Management Review.

# Method—Step-by-Step Prose

Every controlled area displays a laminated Environmental Parameter Card that shows the current specification (for example, “22 °C ± 2 °C, RH 45–55 %, ISO 8 cleanliness, ESD Class 0”). The card cites the drawing or spec that drives each limit.

Operators read the calibrated thermometer–hygrometer (or data-logger display) at the start of each shift and at any setup change. They write the readings, date, and initials in the Environmental Monitoring Log kept beside the traveler. Where data loggers are installed, the daily summary sheet printed from the logger replaces manual entries.

If any reading is outside the posted limit the operator stops the affected operation, secures product completed since the last in-limit reading, and notifies the Process Owner. The Product-Hold section of the log captures batch-ID, time, and nature of the out-of-limit event. Disposition follows the Non-conforming-Output procedure P-870-01.

Quality Technicians check each log once per shift change; if a trend toward the upper or lower limit is seen on three successive readings, they alert Maintenance for a preventive-action HVAC adjustment.

All logs (manual or electronic) are scanned to PDF weekly and stored in the “Environmental Records” folder of the DMS for not less than five years.

# Record of Review

| Review Date | Reviewed By | Summary of Changes | MRM Reference |
| --- | --- | --- | --- |
|  | OQCQMS | New Release | OQCQMS |