# Purpose

Explain how product is preserved, packaged, and protected from the end of production through delivery, storage, and internal transfer, as required by AS9100 Rev D Clause 8.5.4.

# Scope

Covers all finished goods, in-process assemblies awaiting the next operation, spare parts, and returned units handled by [The Organization] or its logistics partners.

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Production Supervisor | Confirms parts meet cleaning & drying requirements before packaging. |
| Warehouse Team | Applies correct packaging level, labels, humidity/FOD controls, and storage location. |
| Quality Control (QC) Inspector | Verifies preservation and labeling at final inspection; audits storage areas. |
| Process Engineer | Maintains Preservation Specification Table (A-854-001). |
| Quality Manager | Trends preservation-related non-conformances; updates this WI when specs change. |

# Definitions

| Term | Definition |
| --- | --- |
| Level 1 Packaging | Basic dust protection & identification for in-house transfers (<24 h). |
| Level 2 Packaging | Static-dissipative bag + desiccant + outer carton for domestic shipping. |
| Level 3 Packaging | Vacuum-sealed barrier bag + desiccant + foam-lined crate for export or long-term storage (>1 yr). |
| FOD | Foreign Object Debris that could damage product integrity. |
| ESD | Electrostatic Discharge control measures (wrist strap, ESD bag, grounded surfaces). |

# Work Instruction

| Step | Action | Responsible | Record / Reference |
| --- | --- | --- | --- |
| 1 | Clean & Dry – Remove machining oils, fingerprints, and debris; allow parts to air-dry or blow-dry with filtered air. | Operator | Traveller / WO |
| 2 | Visual FOD Check – Inspect surfaces, cavities, and threads under proper lighting; remove lint or chips. | Operator | Traveller |
| 3 | Select Preservation Level – Refer to A-854-001; choose Level 1, 2, or 3 based on product type, customer contract, and transit/storage duration. | Warehouse | A-854-001 |
| 4 | Apply Surface Protection – If required, coat with approved rust inhibitor or apply humidity indicator card. | Warehouse | Logbook |
| 5 | Package & Cushion – Use ESD bags, foam inserts, barrier bags, crates per table spec; avoid metal-on-metal contact. | Warehouse | – |
| 6 | Label – Apply part/serial no., revision, quantity, handling symbols (fragile, ESD), and destination info. | Warehouse | Shipping label |
| 7 | QC Verification – QC checks labeling, packaging integrity, moisture cards, and container ID; signs final traveller block. | QC Inspector | Traveller |
| 8 | Store or Ship – Place in designated rack/bin; log location in ERP. For shipment, hand over to Logistics with copy of CoC / 8130-3. | Warehouse | ERP |
| 9 | Monitor Storage Conditions – Warehouse monitors temp/humidity daily; record excursions and initiate corrective action if limits exceeded. | Warehouse | Environmental log |
| 10 | Audit & Review – QC audits random storage lots monthly for FOD, label legibility, and container damage; report findings at MR. | QC Inspector | Audit report |

# Review & Update Frequency

- Annual review or when a new product line, material, or customer requirement is introduced.

# References

- AS9100 Rev D — Clause 8.5.4 Preservation

- P-854-01 Preservation of Product (Procedure)

- WI-853-001 Identification & Traceability

- P-750-01 Control of Documented Information

Documented Information (maintained separately)

• A-854-001 Preservation Specification Table
• Environmental Condition Monitoring Log
• ERP location entries & shipping records
• Traveller/WO with preservation sign-offs
• Storage-condition excursion reports

# Record of Review

| Review Date | Reviewed By | Summary of Changes | MRM Ref. |
| --- | --- | --- | --- |
| – | – | Initial release | – |